TTL     Program to input text into a buffer

BACK_SPACE      EQU     $08               ASCII code for backspace

DELETE          EQU     $01               ASCII code for delete

CARRIAGE_RETURN EQU     $0D               ASCII code for carriage return

ORG     $004000           Data origin

LINE_BUFFER     DS.B    64                Reserve 64 bytes for line buffer

*

*  This procedure inputs a character and stores it in a buffer

ORG     $001000           Program origin

LEA     LINE_BUFFER,A2    A2 points to line buffer

NEXT            BSR     GET_DATA          Call subroutine to get input

CMP.B   #BACK_SPACE,D0    Test for back space

BEQ     MOVE_LEFT         If back space then deal with it

CMP.B   #DELETE           Test for delete

BEQ     CANCEL            If delete then deal with it

CMP.B   #CARRIAGE_RETURN  Test for carriage return

BEQ     EXIT              If carriage return than exit

MOVE.B  D0,(A2)+          Else store input in memory

BRA     NEXT              Repeat

.

.                         Remainder of program

END

****

ORG  $001000         Start of data region

First	DC.B 10,66           The values 10 and 66 are stored in consecutive bytes

DC.L $0A1234         The value $000A1234 is stored as a longword

Date	DC.B 'April 8 1985'  The ASCII characters as stored as a sequence of 12 bytes

DC.L 1,2             Two longwords are set up with the values 1 and 2

****

ORG        $001000      Origin for data

TABLE     DS.W       256          Save 256 words for "TABLE"

POINTER1  DS.L       1            Save one longword for "POINTER1"

POINTER2  DS.L       1            Save one longword for "POINTER2"

VECTOR_1  DS.L       1            Save one longword for "VECTOR_1"

INIT      DC.W       0,$FFFF      Store two constants ($0000, $FFFF)

SETUP1    EQU        $03          Equate "SETUP1" to the value 3

SETUP2    EQU        $55          Equate "SETUP2" to the value $55

ACIAC     EQU        $008000      Equate "ACIAC" to the value $8000

RDRF      EQU        0            RDRF = Receiver Data Register Full

PIA       EQU        ACIAC+4      Equate "PIA" to the value $8004

*

ORG        $018000      Origin for program

ENTRY     LEA        ACIAC,A0     A0 points to the ACIA

MOVE.B     #SETUP2,(A0) Write initialization constant into ACIA

*

GET_DATA  BTST.B     #RDRF,(A0)   Any data received?

BNE        GET_DATA     Repeat until data ready

MOVE.B     2(A0),D0     Read data from ACIA

****

MOVEA.L #ACIA,A0     Load A0 with the address of the ACIA

*                                 (ACIA is an I/O device)

READ_STATUS  MOVE.B  (A0),D0      Place the contents of the location

*                                 pointed at by A0 in D0.

BTST.B  #0,D0        Test bit 0 of D0 for data ready

BEQ     READ_STATUS  Repeat until ACIA ready

MOVE.B  2(A0),D0     Read the input

****

MOVEA.L  #HEAD,A0  A0 initially points to the start of the list

LOOP TST.L    (A0)      IF the address field = 0

BEQ      EXIT         THEN exit

MOVEA.L  (A0),A0      ELSE read the address of the next element

BRA      LOOP      Continue

EXIT MOVEA.L  #NEW,A1   Pick up address of new element

MOVE.L   A1,(A0)   Add new entry to end

CLR.L    (A1)      Add new terminator

****

MOVE.B  #16,D0       Set up a counter for 16 elements

MOVEA.L #BUFFER,A0   A0 points to the first element of the array

LOOP CLR.L   (A0)+        Clear an element and move pointer to the next

SUBQ.B  #1,D0        Decrement the element counter

BNE     LOOP         Repeat until the count is zero

****

TABLE_1       EQU       $002000       Location of Table 1

TABLE_2       EQU       $003000       Location of Table 2

N             EQU       $30           48 elements in each table

.

MOVEA.L   #TABLE_1,A0   A0 points to the top of Table 1

MOVEA.L   #TABLE_2,A1   A1 points to the top of Table 2

MOVE.B    #N,D0         D0 is the element counter

NEXT_ELEMENT  CMPM.B    (A0)+,(A1)+   Compare a pair of elements

BNE       FAIL          If not the same then exit to FAIL

SUBQ.B    #1,D0         Else decrement element counter

BNE       NEXT_ELEMENT  Repeat until all done SUCCESS

.

.                       Deal with success (all matched)

.

FAIL          .                       Deal with fail (not all matched)

***

*     D2.L contains the longword to be printed as 8 hexadecimal characters

*     D1.B is used as a counter to count the 8 characters

*     D0.B is used to send a character to the print subroutine

*     D3.L is used as a temporary register

*     TRANS is the address of the translation table

*

MOVE.B  #8,D1         Eight hexadecimal characters to print

LOOP  ROL.L   #4,D2         Move next nibble to least significant position

MOVE.B  D2,D3         Copy least significant byte to D3

ANDI.L  #$0000000F,D3 Clear all D3 apart from least significant nibble

MOVEA.L D3,A0         A0 now contains least significant nibble

MOVE.B  TRANS(A0),D0  Read the ASCII character from the table

BSR     PRINT_CHAR    Display it

SUB.B   #1,D1         Subtract 1 from loop counter

BNE     LOOP          Repeat until all eight chars printed

.

.

.

TRANS DC.B    '0123456789ABCDEF' Translation table as ASCII string

*****

m   EQU      <m>		Number of rows (1 to m)

n   EQU      <n>		Number of columns (1 to n)

A   EQU      <address>		Start address of array A

B   EQU      <address>		Start address of array B

C   EQU      <address>		Start address of array C

MOVEA.L  #A,A0		A0 points to base of matrix A

MOVEA.L  #B,A1		A1 points to base of matrix B

MOVEA.L  #C,A2		A2 points to base of matrix C

CLR.W    D2		Clear the element offset

MOVE.W   #m,D0  		D0 is the row counter

L2  MOVE.W   #n,D1		D1 is the column counter

L1  MOVE.B   (A0,D2.W),D6		Get element from A

ADD.B    (A1,D2.W),D6		Add element from B

MOVE.B   D6,(A2,D2.W)		Store sum in C

ADDQ.W   #1,D2		Increment element pointer

SUB.W    #1,D1		Repeat until n columns added

BNE      L1

SUB.W    #1,D0		Repeat until m rows added

BNE      L2

****

MOVEA.L   #A,A0	A0 points to base of array A

SUBQ.L    #1,D0	Rows are numbered from 1

MULU      #n,D0	D0 now contains the row offset

SUBQ.L    #1,D1	Columns are numbered from 1

ADD.L     D1,D0	D0 now contains column + row offset

ADDA.L    D0,A0	A0 now points to the i,j th element

****

P1  DS.W    1         Parameter P1 (one word)

.

PEA     P1        Push the address of P1 on the stack

BSR     ABC       Call subroutine 'ABC'

LEA     4(A7),A7  Increment stack pointer to remove address of P1

.

.

ABC LEA     4(A7),A0  Load the address of P1 on the stack into A0

MOVEA.L (A0),A0   Read the address of P1 on the stack

MOVE.W  (A0),D0   Read the value of P1

.

.

****

CLR.L  D1        Clear D1 as DIVU requires a 32-bit dividend

MOVE.B D0,D1     Copy the source to D1

DIVU   #100,D1   Divide D1 by 100 to get 100's digit in D1(0:15)

MOVE.W D1,D0     Save the 100's digit in D0 (in least significant position)

SWAP   D1        Move remainder in D1(15:31) to low-order word

AND.L  #$FFFF,D1 Clear most significant word of D1 before division

DIVU   #10,D1    Divide remainder by 10 to get 10's digit in D1(0:15)

LSL.W  #4,D0     Move 100's digit in result one place left

OR.W   D1,D0     Insert 10's digit into the result

LSL.W  #4,D0     Move both 100's and 10's digits one place left

SWAP   D1        Move remainder in D1(15:31) to low-order word

OR.W   D1,D0     Insert 1's digit in least significant place

****

MOVE.W  #$04,CCR      Clear the X-bit and set the Z-bit

NEGX.L  X_low         Negate the low-order longword

NEGX.L  X_high        Negate the high-order longword

****

DIVS    D0,D1      Divide the 32-bit value in D1 by the word in D0

EXT.W   D1         Sign-extend the 16-bit quotient in D1.W to 32 bits

****

MOVE.W  #5,D0          Six bytes in the string to be added

MOVE.W  #4,CCR         Clear X-bit of CCR and set Z-bit

LEA     String1+1,A0   A0 points at source string

LEA     String2+1,A1   A1 points at destination string

LOOP	ABCD    -(A0),-(A1)    Add a pair of digits (with any carry-in)

DBRA    D0,LOOP        Repeat until 6 bytes (12 digits) added

****

*  D0 = input/output register (only D0.B modified)

*  D1 = one's counter (not modified by subroutine)

*  D2 = pointer to bit of D0 to be tested (not modified by subroutine)

*

ONES_COUNT MOVEM.L  D1-D2,-(A7)      Save working registers

CLR.B    D1               Clear one's counter

MOVEQ    #7,D2            D2 initially points at msb of D0.B

*

NEXT_BIT   BTST     D2,D0            Test the D2.th bit of D0

BEQ      LOOP_TEST        If zero then nothing more to do

ADDQ.B   #1,D1                    else increment 1's count

LOOP_TEST  SUBQ.B   #1,D2            Decrement bit pointer

BGE      NEXT_BIT         Repeat until count negative

MOVE.B   D1,D0            Transfer one's count to D0

MOVEM.L  (A7)+,D1-D2      Restore working registers

RTS                       Return

****

BLOCK1	EQU    <address 1>

BLOCK2	EQU    <address 2>

SIZE	EQU    <number of words in block>

*

LEA    BLOCK1,A0	A0 points to first block

LEA    BLOCK2,A1	A1 points to second block

MOVE.W #SIZE-1,D0	D0 is the word counter

*

LOOP	CMPM.W (A0)+,(A1)+	Compare a pair of words

BNE    NOT_SAME	IF not same THEN exit

DBRA   D0,LOOP	            ELSE repeat until all done

*

ALL_SAME	exit here if they are the same

.

.

NOT_SAME	exit here if they are different

****

MOVE.B  HEX,D0      Get HEX value to be converted into D0

ADDI.B  #$30,D0     Add $30 to it

CMPI.B  #$39,D0     Test for hex values in the range $0A to $0F

BLS     EXIT        If not in range $0A to $0F then exit

ADDQ.B  #$07,D0                                Else add 7

EXIT MOVE.B  D0,CHAR     Save the result in CHAR

****

CASE         MOVE.B TEST,D0       Put the value of TEST in D0

BEQ    ACT1          If zero then carry out ACT1

SUBQ.B #1,D0         Decrement TEST

BEQ    ACT2          If zero then carry out ACT2

SUBQ.B #1,D0         Decrement TEST

BEQ    ACT3          If zero then carry out ACT3

EXCEPTION    ...                  Else deal with the exception

BRA    EXIT          Leave CASE

ACT1         ...                  Execute action 1

BRA    EXIT          Leave CASE

ACT2         ...                  Execute action 2

BRA    EXIT          Leave CASE

ACT3         ...                  Execute action 3

...

...                  (Fall through to exit)

EXIT         ...                  Single exit point for CASE

****

LEA     Table,A0		A0 points at the table of longword pointers

MULU    #4,D0		Multiply the offset in D0 by 4

MOVEA.L (A0,D0),A0	Read the pointer from the table

JMP     (A0)		Jump to the computed routine

.

.

Table  DC.L  Code_1		Pointers to the routines

DC.L  Code_2

.

.

DC.L  Code_N

Code_1 .				Code for routine 1

.

.

JMP   Exit			Jump to common exit point

****

MOVE.W  #255,D1         Setup D1 as a counter with maximum block size 256

WAIT BTST.B  #0,READY        Test bit 0 of READY

BEQ     WAIT            Repeat test until not zero

MOVE.W  INPUT,D0        Get input and move it to D0

.

.                       Process input

.

CMPI.W  #$FFFF,D0       Test input for terminator

DBEQ    D1,WAIT         Continue for 256 cycles or until true

****

BSR     GET_DATA   Get input to be tested in D0

CLR.B   FLAG       Clear FLAG

TST.L   D0         Test result

BMI.S   NEXT       If zero then exit with FLAG = 0

MOVE.B  #$FF,FLAG  Else set all the bits of FLAG

NEXT    ...                Continue

****

BSR     GET_DATA   Get input to be tested in D0

TST.L   D0         Test result

SPL.B   FLAG       If negative then FLAG = 0 else FLAG = $FF

****

Sub2  .

.

BEQ  Exit       Detect a problem here

.

.

RTS             Normal return point to Sub1

Exit  LEA  4(A7),A7   Move past the normal return point

RTS             and return directly to main program

****

BSR  SubA        Call subroutine A

.

.

SubA  .                Subroutine A

.

BSR SubB         Call subroutine B

BCC OK           IF no error in subroutine B THEN continue

RTS                                          ELSE return

OK    .

.

RTS              Normal return point from subroutine A

SubB  .                Subroutine B

.

.

ORI  #%00001,CCR Escape from here - Set C-bit for error

BRA  Rtrn        Now exit

.

.

Rtrn  RTS              Normal return

****

* Calculator program

*

BSR Initialize   Set up the constants

Again   BSR GetNum1      Read the first number

BSR GetOpr       Read the operator: one of +,-,*,/

BSR GetNum2      Read the second number

BSR Calc         Perform the calculation

BSR PrintRes     Display the result

.

.

GetNum1 BSR GetDigit     Get a digit from the keyboard

BSR Valid        Test for a valid digit in range 0 to 9

BCS DigErr       IF not valid digit THEN deal with error

BSR SaveDigit                       ELSE save it

.

RTS

****

.

BSR     GET_DATA

.

.

GET_DATA MOVE.W  CCR,-(A7)          Save CCR on stack

MOVEM.L D1-D7/A0-A6,-(A7)  Save working registers on stack

.

.

MOVEM.L (A7)+,D1-D7/A0-A6  Restore working registers

RTR                        Restore CCR and return

****

SIZE  EQU    <size>          The size of the array TABLE in bytes

Table DS.B   SIZE            Save SIZE bytes for the array TABLE

LOWER DS.L   1               Reserve longword for lower bound of TABLE

UPPER DS.L   1               Reserve longword for upper bound of TABLE

.

.

*                            Set up array bounds

LEA    Table,A0        A0 points to the start of the array

MOVE.L A0,LOWER        Store the lower bound of the array

LEA    (SIZE-1,A0),A0  Calculate the upper bound of the array

MOVE.L A0,UPPER        Store the upper bound of the array

.

.

*                            Perform boundary tests

LEA    Table,A0        A0 points to the start of the array

LEA    (A0,D0.W),A0    A0 points to the jth element

CMP2   LOWER,A0        Test for out of bounds element

BCS    ERROR           IF carry set THEN error

.                                   ELSE continue

.

ERROR .                      Deal with error condition

****

FIELDS DS.L    64

*      Assume that the sector map is made up of 64 longwords

*      (this assumption means that we can use 32-bit fields)

*

LEA     FIELDS,A0       A0 points to sector map

MOVE.W  #63,D7          Up to 64 bit fields to search

LOOP   BFFFO   (A0){0:32},D1   Look for a free sector

LEA     4(A0),A0        Increment pointer to next bit field

CMP.B   #32,D1          IF D1 = 32 THEN no free sector found

BNE     FOUND           IF D1 not 32 THEN free sector found

DBRA    D7,LOOP         REPEAT UNTIL all 64 fields tested

BRA     FULL            IF here THEN no free sector located

FOUND  LEA     -4(A0),A0       Wind back the pointer

BFCLR   (A0){D1:1}      Clear bit to claim sector

.

.

FULL   deal with disk full

****

CLR.L   D0               Initial bit field offset = 0

LEA     FIELDS,A0        A0 points at sector bitmap

MOVE.W  #64-1,D7         Up to 64 bit fields to test

LOOP   BFFFO   (A0){D0:32},D0   Look for free sector, IF found Z = 0 and

*                                bit field offset from (A0) is loaded

*                                into D0, ELSE Z = 1 and bit field offset

*                                plus 32 is loaded into D0.

DBEQ    D7,LOOP          Decrement counter until Z = 0 or all 64

*                                 fields searched without success.

BEQ     FULL             IF Z = 1, no 1's found and disk is full

BFCLR   (A0){D0:1}       ELSE claim sector. D0 = sector number

****

MOVE.W #5,D0              Six lines to move

LOOP	BFEXTU (A0){4:15},D1      Copy a line of the image to D1

BFINS  D1,80(A0){7:15}    Copy it to its destination

LEA    8(A0),A0           Update the pointer by 8 bytes (one line)

DBRA   D0,LOOP            Repeat until all lines moved

****

*     Read the packed variable from memory and store it in bits 0 - 2 of D0

*

LOAD  MOVE.W #$0380,D0 Load the mask word 0000001110000000

AND.W  <ea>,D0   Packed word is masked to bits 7 - 9 in D0

LSR.W  #7,D0     Justify D0 to get bits in least significant position

RTS

*

*     The 3-bit variable to pack is initially in bits 0 - 2 of D0 and

*     is to be packed in bit 7 - 9 of the specified memory location.

*     Note that the store operation must not modify any other bits

*     of the packed word (i.e., bits 0 - 6 and 10 - 15)

*

STORE LSL.W  #7,D0     Shift bit field to be stored to bits 7 - 9

MOVE.W #$FC7F,D1 Load mask word 1111110001111111 in D1

AND.W  <ea>,D1   Get packed word in D1, clear bits 7 - 9

OR.W   D0,D1     Insert bit field from D0 in bits 7 - 9 of D1

MOVE.W D1,<ea>   Store packed word in memory

RTS

****

*                      [<ea>]=0101010101010100 (initial packed string)

LOAD  MOVE.W #$0380,D0 D0=0000001110000000 (mask bits = bits 7 to 9)

AND.W  <ea>,D0   D0=0000000100000000 (mask D0 to bits 7 - 9)

LSR.W  #7,D0     D0=0000000000000010 (left justify bit field)

RTS

****

STORE LSL.W  #7,D0     D0=0000001100000000 (move variable to bits 7-9)

MOVE.W #$FC7F,D1 D1=1111110001111111 (mask to clear bits 7 - 9)

AND.W  <ea>,D1      1010101010101010 (data before packing)

*                      D1=1010100000101010 (clear bits 7 - 9)

OR.W   D0,D1     D1=1010101100101010 (insert bits 7 - 9 from D0)

MOVE.W D1,<ea>   Store packed word in memory

RTS

PTR	EQU      <pointer to record offset>

LSL.L    #2,D0			Multiply the student index by 4

LEA      (PTR,A0,D0.L),A1	Calculate address of pointer to record

MOVEA.L  (A1),A1		Read the actual pointer

ADDA.L   #3,A1			Calculate address of CS result

MOVE.B   (A1),D1		Read the result.

****

*      IF L THEN S

*

TST.B  D0        Test the lower order byte of D0

BEQ.S  EXIT      If zero then exit, else continue

S                Action S

EXIT

**********************************************************************

*      IF L THEN S1 ELSE S2

*

TST.B  D0        Test the lower order byte of D0

BEQ.S  ELSE      If zero then S2 (ELSE part)

S1               If not zero then S1 (THEN part)

BRA.S  EXIT      Skip past ELSE part

ELSE   S2               Action S2 (ELSE part)

.

.

.

EXIT

************************************************************

*      FOR I = N1 TO N2

*

MOVE.B #N1,D0    D0 is the loop counter

NEXT   S                Action S (body of loop)

ADDQ.B #1,D0     Increment loop counter

CMP.B  #N2+1,D0  Test for end of loop

BNE    NEXT      Repeat until counter = N2 + 1

EXIT

************************************************************

*      WHILE L DO S

*

REPEAT TST.B  D0        Test the lower order byte of D0

BEQ.S  EXIT      If zero then quit loop

S                Body of loop (Action S)

BRA    REPEAT    Repeat

EXIT

************************************************************

*      REPEAT S UNTIL L

*

NEXT   S                Body of loop (Action S)

TST.B  D0        Test the lower order byte of D0

BEQ.S  NEXT      Repeat until true

EXIT

************************************************************

*      FOR I = N DOWNTO -1 (Using the DBRA instruction)

*

MOVE.W #N,D1     D1 is the loop counter

NEXT   S                Action S (body of loop)

DBRA   D1,NEXT   Decrement D1 and loop if not -1