* ACIA_Initialize and Character_Input routine

* Data register D0 contains Function (zero=initialize, non-zero = get a character)

* Data register D0 is re-used for the Cycle_Count (a timeout mechanism)

* Data register D1 returns Error_Status

* Data register D2 returns the character from the ACIA

* Data register D3 is temporary storage for the ACIA’s status

* Data register D4 is temporary storage for the masked ACIA’s status (error bits)

* Address register A0 contains the address of the ACIA’s control/status register

*

Char_In MOVEM.W D3-D4,-(A7)   Push working registers on the stack

CLR.B   D1            Start with Error_Status clear

CMP.B   #0,D0         IF Function not zero THEN get input

BNE     InPut                              ELSE initialize ACIA

MOVE.B  #3,(A0)       Reset the ACIA

MOVE.B  #$19,(A0)     Configure the ACIA

BRA     Exit_2        Return after initialization

*

InPut   MOVE.W  #$FFFF,D0     Set up Cycle_Count for time-out (reuse D0)

InPut1  MOVE.B  (A0),D3       Read the ACIA’s status register

MOVE.B  D3,D4         Copy status to D4

AND.B   #%01111100,D4 Mask status bits to error conditions

BNE     Exit_1        IF status indicates error, set error flags & return

BTST    #0,D3         Test data_ready bit of status

BNE     Data_Ok       IF data_ready THEN get data

SUBQ.W  #1,D0                       ELSE decrement Cycle_Count

BNE     InPut1        IF not timed out THEN repeat

MOVE.B  #$FF,D1                        ELSE Set error flag

BRA     Exit_2                             and return

*

Data_Ok MOVE.B  (2,A0),D2     Read the data from the ACIA

BRA     Exit_2        and return

*

Exit_1  MOVE.B  D4,D1         Return Error_Status

Exit_2  MOVEM.W (A7)+,D3-D4   Restore working registers

RTS                   Return

****

LEA     (-4,A7),A7     Save space on stack for Error_Status and Input_Char

MOVE.L  #ACIA,-(A7)    Push ACIA address on the stack

MOVE.W  Func,-(A7)     Push function code on the stack

BSR     Char_In        Call subroutine

LEA     (6,A7),A7      Clean up stack - remove parameters Function/ACIA

MOVE.W  (A7)+,Char     Pull the input character off the stack

MOVE.W  (A7)+,Err      Pull the Error_Status off the stack

* Character_Input and ACIA_Initialize routine

* Data register D3 is temporary storage for the ACIA’s status

* Data register D4 is temporary storage for the Cycle_Count

* Address register A0 contains the address of the ACIA’s control/status register

*

Char_In MOVEM.L A0/D3-D4,-(A7) Push working registers on the stack

MOVE.L  (18,A7),A0     Read address of ACIA from the stack

CLR.B   (24,A7)        Start with Error_Status clear

CMPI.B  #0,(16,A7)     IF Function not zero THEN get input

BNE     InPut                               ELSE initialize ACIA

MOVE.B  #3,(A0)

MOVE.B  #$19,(A0)

BRA     Exit_2         Return after initialization

*

InPut   MOVE.W  #$FFFF,D0      Set up Cycle_Count for time-out (reuse D0)

InPut1  MOVE.B  (A0),D3        Read the ACIA’s status register

MOVE.B  D3,D4          Copy status to D4

AND.B   #%01111100,D4  Mask status bits to error conditions

BNE     Exit_1         IF status indicates error, deal with it

BTST    #0,D3          Test data_ready bit of saved status status

BNE     Data_OK        IF data_ready THEN get data

SUBQ.W  #1,D0                        ELSE decrement Cycle_count

BNE     InPut1         IF not timed out THEN repeat

MOVE.B  #$FF,(24,A7)                    ELSE Set error flag

BRA     Exit_2                          and return

*

Data_OK MOVE.W  (2,A0),(22,A7) Read the data from the ACIA and put on the stack

BRA     Exit_2         and return

*

Exit_1  MOVE.B  D4,(24,A7)     Return Error_Status

Exit_2  MOVEM.L (A7)+,A0/D3-D4 Restore working registers

RTS                    Return

PEA     Func           Push Function address on the stack

PEA     ACIA           Push ACIA address on the stack

PEA     Error_Status   Push address of Error_Status

PEA     Char           Push address of input data

BSR     Char_In        Call subroutine

LEA     (16,A7),A7     Clean up the stack - remove the four addresses

* Character_Input and ACIA_Initialize routine

* Data register D0 is temporary storage for the timeout counter

* Data register D3 is temporary storage for the ACIA’s status

* Data register D4 is temporary storage for the Cycle_Count

* Address register A0 points at the location of the character input from the ACIA

* Address register A1 points at the location of the Error_Status

* Address register A2 points at the location of the ACIA

* Address register A3 points at the location of the Function code

*

Char_In MOVEM.L A0-A3/D0/D3-D4,-(A7) Push working registers on the stack

MOVEA.L (32,A7),A0     Read address of Char from the stack

MOVEA.L (36,A7),A1     Read address of Error_Status from the stack

MOVEA.L (40,A7),A2     Read address of ACIA from the stack

MOVEA.L (44,A7),A3     Read address of Function from the stack

CLR.B   (A1)           Start with Error_Status clear

CMPI.B  #0,(A3)        IF Function not zero THEN get input

BNE     InPut                               ELSE initialize ACIA

MOVE.B  #3,(A2)

MOVE.B  #$19,(A2)

BRA     Exit_2         Return after initialization

*

InPut   MOVE.W  #$FFFF,D0      Set up Cycle_Count for timeout

InPut1  MOVE.B  (A2),D3        Read the ACIA’s status register

MOVE.B  D3,D4          Copy status to D4

AND.B   #%01111100,D4  Mask status bits to error conditions

BNE     Exit_1         IF status indicates error, set flags and return

BTST    #0,D3          Test data_ready bit of status

BNE     Data_OK        IF data_ready THEN get data

SUBQ.W  #1,D0                        ELSE decrement Cycle_Count

BNE     InPut1         IF not timed out THEN repeat

MOVE.B  #$FF,(A1)                       ELSE Set error flag

BRA     Exit_2                          and return

Data_OK MOVE.W  (2,A2),(A0)    Read the data from the ACIA

BRA     Exit_2

*

Exit_1  MOVE.B  D4,(A1)        Return Error_Status

Exit_2  MOVEM.L (A7)+,A0-A3/D0/D3-D4 Restore working registers

RTS

****

PEA     Char           Push the address of the destination for the input

PEA     Error_Status   Push the address of the Error_Status message

PEA     ACIA           Push the ACIA’s address on the stack

MOVE.W  Function,-(A7) Push the value of function code on the stack

BSR     Char_In        Call the subroutine

LEA     (14,A7),A7     Clean up the stack - remove the four parameters

* Character_Input and ACIA_Initialize routine

* Stack frame location A6 - 6 holds the ACIA’s status

* Stack frame location A6 - 4 holds the ACIA’s masked status (error bits only)

* Stack frame location A6 - 2 holds the Cycle_Count

* Address register A1 contains the address of the Error_Status

* Address register A2 contains the address of the ACIA’s control/status register

*

Char_In LINK    A6,#-6         Create a stack frame for three words

MOVEM.L A1-A2,-(A7)    Push working registers on the stack

MOVEA.L (14,A6),A1     Read address of Error_Status from the stack

MOVEA.L (10,A6),A2     Read address of ACIA from the stack

CLR.B   (A1)           Clear Error_Status

MOVE.W  #$FFFF,(-2,A6) Set up Cycle_Count for timeout

CMPI.B  #0,(8,A6)      IF Function not zero THEN get input

BNE     InPut                               ELSE initialize ACIA

MOVE.B  #3,(A2)        Reset ACIA

MOVE.B  #$19,(A2)      Configure ACIA

BRA     Exit_2         Return after initialization

*

InPut   MOVE.B  (A2),(-4,A6)   Read the ACIA’s status register - save in Temp1

MOVE.B  (-4,A6),(-6,A6) Copy status to Temp2

ANDI.B  #%01111100,(-6,A6) Mask status bits to error conditions

BNE     Exit_1         IF status indicates error, set flag and exit

BTST    #0,(-4,A6)     ELSE Test data_ready bit of status

BNE     Data_OK        IF data_ready THEN get data

SUBQ.W  #1,(-2,A6)                   ELSE decrement Cycle_Count

BNE     InPut          IF not timed out THEN repeat

MOVE.B  #$FF,(A1)                       ELSE Set error flag

BRA     Exit_2                          and return

*

Data_OK MOVE.L  (18,A6),A1     Get address for data destination (re-use A1)

MOVE.B  (2,A2),(A1)    Read the data from the ACIA

BRA     Exit_2

*

Exit_1  MOVE.B  (-6,A6),(A1)   Return Error_Status

Exit_2  MOVEM.L (A7)+,A1-A2    Restore working registers

UNLK    A6             Collapse the stack frame before returning

RTS

****